Data-Driven Prediction of Unknown Bridge Foundation Depth Using Nondestructive Testing and Machine Learning

Project Proposal

CE 5316 - MACHINE LEARNING FOR CE

By

Md Manshib Tazowar

Graduate Student

Student ID: 1002233352

Department of Civil Engineering

The University of Texas at Arlington

To

Dr. Jinzhu Yu

Assistant Professor

Department of Civil Engineering

Department of Industrial, Manufacturing, and Systems Engineering,

The University of Texas at Arlington

April 2026

ABSTRACT:

There are over 643,000 bridges across the United States where 65,000 were classified as having unknown foundations. Unknown foundation bridges pose critical challenges to Departments of Transportation (DOTs) particularly for assessing scour vulnerability and planning maintenance or retrofits. Traditional methods such as excavation or boring are costly, invasive, and impractical for broad deployment, highlighting the need for efficient data-driven alternatives. This study proposes a machine learning (ML) framework to predict unknown bridge foundation depth using field data obtained from Sonic Echo Impulse Response (SEIR), which is a non-destructive testing method. Structural and site-specific parameters such as built year, geometry, soil conditions, and hydraulic environment will be combined with SEIR test data to develop predictive models. Multiple supervised algorithms, including Random Forest, Support Vector Machine (SVM), XGBoost, and LightGBM, will be trained and validated using known-foundation bridge data. Model accuracy will be benchmarked using different statistical matrices. The resulting model will provide a scalable and interpretable approach to estimate foundation depth, improving infrastructure risk assessment and supporting data-informed asset-management decisions.

### PROJECT PLAN

Dataset:
The dataset will include SEIR test results from approximately 50-100 bridges with known foundation depths and original field-testing data, supplemented by Texas Department of Transportation (TxDOT), National Bridge Inventory (NBI) and Info Bridge data. Each record will contain geometric, structural, material, and hydraulic parameters associated with the tested bridges.

Data Processing:

Processing steps will include cleaning and normalization, encoding categorical variables, and feature selection using correlation and permutation importance analysis. Principal Component Analysis (PCA) may be used for dimensionality reduction.

Tools and Languages:

Python will be used for all stages machine learning and modeling (Pandas, Scikit-learn, PCA, Regression Models, Boosting Models, Neural Networks, Matplotlib etc.).

Problem Formulation:

The primary goal is to predict bridge foundation depth as a continuous output using multivariate input data. The target variable (y) represents measured foundation depth (ft) obtained from SEIR testing, while the input feature matrix (x) includes different structural, geotechnical and hydraulic attributes such as bridge geometry (e.g., span length, roadway width, skew angle, number of spans), waterway opening, soil formation and type.

This will be modeled as a supervised regression problem, where algorithms learn the mapping f(x) → y. Models will be trained using cross-validation and evaluated through R2, MAE, RMSE, and RRMSE to quantify predictive performance and generalization. Additionally, feature-importance analysis will be conducted to identify which parameters most strongly influence foundation depth.

A complementary classification analysis may also categorize foundation types (drilled shaft, abutment, shallow footing) using the same feature set to examine model adaptability to discrete outcomes.

Project Type:

Research-oriented project focused on developing and validating a predictive modeling framework integrating NDT-based field data with machine learning techniques.